Abstract Template

Alternative Energy Sources, Materials & Technologies (AESMT’22), Volume XX, (pp. XX - XX) 2022

In-situ measurements of ground thermal properties in Plovdiv, Bulgaria

A. G. Georgiev1, R. K. Popov2*, D. B. Dzhonova-Atanasova3

1Technical University of Sofia, Plovdiv Branch, Dept. of Mechanics,25 Tsanko Diustabanov Str, 4000 Plovdiv, Bulgaria

2Plovdiv University “Paisii Hilendarski”, Department of Electronics, Telecommunications and Information Technologies (ETIT), 24 Tsar Assen Str, 4000 Plovdiv, Bulgaria, rum_pop@yahoo.com

3 Institute of Chemical Engineering, Bulgarian Academy of Sciences, Acad. G. Bonchev Str., Bl. 103, 1113, Sofia, Bulgaria

The knowledge of the geodata (subsurface characteristics) is important for the design and construction of the Underground Thermal Energy Storage (UTES) and to design an installation using Borehole Thermal Energy Storage (BTES) - it is a big advantage during the project calculations and the construction of the geothermal systems. In situ determination of ground thermal conductivity, borehole thermal resistance and undisturbed soil temperature can be done by installing a vertical borehole heat exchanger (BHE) and performing the so-called Thermal Response Test (TRT).

Keywords: Thermal Response Test, ground thermal properties, borehole heat exchangers

Manuscript Template

Alternative Energy Sources, Materials & Technologies (AESMT’22), Volume XX, (pp. XX - XX) 2022

In-situ measurements of ground thermal properties in Plovdiv, Bulgaria

A. G. Georgiev1, R. K. Popov2*, D. B. Dzhonova-Atanasova3

1Technical University of Sofia, Plovdiv Branch, Dept. of Mechanics,25 Tsanko Diustabanov Str, 4000 Plovdiv, Bulgaria

2Plovdiv University “Paisii Hilendarski”, Department of Electronics, Telecommunications and Information Technologies (ETIT), 24 Tsar Assen Str, 4000 Plovdiv, Bulgaria, rum_pop@yahoo.com

3 Institute of Chemical Engineering, Bulgarian Academy of Sciences, Acad. G. Bonchev Str., Bl. 103, 1113, Sofia, Bulgaria

The knowledge of the geodata (subsurface characteristics) is important for the design and construction of the Underground Thermal Energy Storage (UTES) and to design an installation using Borehole Thermal Energy Storage (BTES) - it is a big advantage during the project calculations and the construction of the geothermal systems. In situ determination of ground thermal conductivity, borehole thermal resistance and undisturbed soil temperature can be done by installing a vertical borehole heat exchanger (BHE) and performing the so-called Thermal Response Test (TRT).

Keywords: Thermal Response Test, ground thermal properties, borehole heat exchangers

© 2022 Bulgarian Academy of Sciences, Union of Chemists in Bulgaria

INTRODUCTION

The Underground Thermal Energy Storage (UTES) is a good solution for saving energy. Laboratory methods have been used to study the ground properties.

MOBILE SYSTEM FOR EXAMINING GROUND THERMAL PROPERTIES

The basic quality requirements for the experimental apparatus are an uninterruptible supply of constant heat power to the borehole and the capability to take highly accurate temperature and flow rate measurements. Fig.1 shows the unit design.

Fig.1. System set-up [2]

Charging and discharging of hybrid installation with solar collectors and BTES

A hybrid installation with solar collectors and Borehole Thermal Energy Storage (BTES) was set up in 2003 at the "Solar Energy Laboratory" of the Technical University Federico Santa Maria (UTFSM) in Valparaiso, Chile with the participation of Prof. A. Georgiev [1].

1. Construction of the system

The installation is located in Plovdiv, Bulgaria, at the campus of the Technical university of Sofia, on the 4-th floor. The main components of the installation are: solar collectors (SC), a hot-water storage (200 l), a latent heat storage (300 l), a water-water heat pump (inverter type), a cold-water storage (150 l), two borehole heat exchangers (BHEs), two expansion tanks, a flat panel heat exchanger and a fan-coil convector [2].

2. Measurement and control requirements

The system complexity demands a lot of measurement points for a long period of time in order to obtain a suitable data set that allows correct thermal and energy process analyses. The number of measurement points can be approximated depending on which specific sensor or instrument type was used.

The operator console is designed as a PC-based station linked via USB or wireless Internet connection to a DAQ board. It ensures that the experiment is being remotely monitored and controlled in real time [3].

3. Different regimes for system operation

Seven different regimes are possible for system operation:

- Mode 1 (charging of the water storages);

- Mode 2 (charging of the borehole storage);

- Mode 3 (direct solar heating);

- Mode 4 (ground-source heat pump heating).

Preparation of the Borehole Heat Exchangers (BHEs)

Polyethylene (PE) pipes are used in the geothermal drills.  The chosen polyethylene pipes are of the PE 100 type and their technical specifications are given in Tab.1.

Table 1. Specifications of PE 100 pipes

----------------------------------------------------------------------------

Name        Surface   Free 	Size of 	Nominal	Hydraulic

area       volume	lamellas 	Diameter	Diameter

----------------------------------------------------------------------------

m²         m³            mm           mm          mm

----------------------------------------------------------------------------

IMTP 25	242.8	   97.1		 2.0	  18.6	  16.0

IMTP 40	171.6	   96.7		 3.1	  26.5	  22.5

IMTP 50	107.1	   97.8		 4.1	  37.5	  36.5

IMTP 70	66.1	   98.5		 4.1	  61.0	  59.6

----------------------------------------------------------------------------

Response analysis

Basic equation

The Kelvin's line source theory [4-6] is normally used as a base for mathematical evaluation of the Thermal Response Test:

ΔT(rb,t)=q4πλr2at∞e−β2βdβ	(1)

where:

ΔT(rb,t)- temperature rise, K;

q - heat injection power per unit borehole length, W/m;

λ - ground thermal conductivity, W/mK;

t – time, s;

a - thermal diffusivity, m²/s;

r - radius from the borehole, m;

β - integration variable, -;

rb- radius of the borehole, m.

Slope determination technique

With this approximation, the delivered heat is considered as coming from a line source, the borehole in Eq,(1). The value of c is to be determined from the inclination of the line in the plot of the mean fluid temperature versus ln(t) [7].

CONCLUSIONS

Some TRTs have been conducted in 2009, 2011 and 2012 at the Technical University of Sofia, Plovdiv Branch, with the created mobile installation. There are some main conclusions.

ACKNOWLEDGEMENTS

This work has been supported financially by the research project 152ПД0045-24/ 26.03.2015 of the Technical University of Sofia, which is gratefully acknowledged by the author.

NOMENCLATURE

q - heat injection power per unit borehole length, W/m;

r - radius, m;

t - time, s;

T - temperature, K;

a - thermal diffusivity, m²/s;

β - integration variable, -;

λ - thermal conductivity, W/mK;

rb- radius of the borehole, m.

Subscripts

b - borehole.

REFERENCES

- [1] Journals G. Florides, S. Kalogirou. First in situ determination of the thermal performance of a U-pipe borehole heat exchanger, in Cyprus. Applied Thermal Engineering 28, 157-163 (2008).

- [2] Proceedings (Transactions, Book of abstracts, Proceedings on CD, etc.) A. Georgiev, R. Popov, S. Tabakova. First Thermal Response Test in Bulgaria. Proc. of Effstock 2009, 11th int. conf. on thermal energy storage, Stockholm, 24.pdf (2009).

- [3] Thesis C. Eklöf, S. Gehlin. TED - a mobile equipment for thermal response test. Master’s Thesis 1996:198E. Sweden: Luleå University of Technology, 1996.

- [4] Books E. Michaelides. Alternative energy sources. Green Energy and Technology, Springer, 2012.

- [5] Chapters Y. Sinak. Models and Projections of Energy Use in the Soviet Union, in: International Energy Economics, T. Steiner (ed.), Chapman and Hall, London, 1998, 1-53.

- [6] Patent J. E. Kidwell, M. L. Fraim. Method of and apparatus for transferring heat energy between a heat exchanging subsystem. US Patent 8,161,759, B2 (2012).

- [7] Literature or data on web sites and documents without authors Council of Industrial Boiler Owners, http://www.cibo.org.

WORD TEMPLATE FOR SUBMISSION OF MANUSCRIPTS

General instructions:

The manuscript should be between 6 and 8 pages (about 3000 words), including photographs, tables, drawings, formulae, etc. Use the template enclosed above to create a manuscript. Open the template file, select the sample title and type over your title, thus the font and size shall be preserved in exact accordance with the layout requirements. The same procedure applies to the authors' names, their e-mail and institution as well of the text in the body, conclusions and references. The text should be typed on A4 format paper using Times New Roman.

Each paper should be written according to following order:

- title;

- author(s);

- affiliation(s);

- abstract;

- keywords;

- body of the text without numerated sections and numerated subsections;

- conclusions;

- acknowledgment (if necessary);

- nomenclature (if necessary);

- list of abbreviations (if necessary);

- references.

Here are additional instructions for all the article parts:

1. Title:

- font size 14;

- centred;

- abbreviations, symbols, chemical formulas, references and footnotes should be avoided.

2. Author(s):

- font size 12;

- centred;

- initials of personal (first) and middle (eventual) names, full surname(s);

- correspondence author should be indicated by asterisk (*).

3. Affiliation(s):

- font size 10;

- centred;

- italic;

- content: institution number, institution name, address, country;

- e-mail only of the correspondence author.

4. Abstract:

- font size 9;

- the abstract should be self-explanatory and intelligible without any references to the text;

- it should contain no more than 250 words.

5. Keywords:

- font size 10;

- no more than six.

6. Body of the text:

- font size 11;

- NOT numerated headings of sections, capital letters, centred;

- NOT numerated sub-headings of subsections, italic, centred;

- numerated sub-sub-headings of sub-subsections, bold;

- manuscripts should be subdivided into labelled sections, e.g. Introduction, Theory, Experiments, Results, Discussion, etc.;

- only Latin and Greek alphabet must be used in preparing the whole manuscript.

7. Conclusions:

- font size 11;

- NOT numerated section heading, capital letter, centred.

8. Acknowledgment (if necessary):

- font size 11;

- NOT numerated section heading, capital letter, centred.

9. Nomenclature (if necessary):

- font size 11;

- NOT numerated section heading, capital letter, centred;

- the variables in nomenclature have to be written in alphabetical order (first Latin and after that Greek alphabet);

- the units should be separated with comma;

- the units should be given by W/mK (not W/m-1K-1);

- subscripts: NOT numerated sub-headings of subsections, italic, centred;

- variables: italic.

10. List of abbreviations (if necessary):

- font size 11;

- NOT numerated section heading, capital letter, centred.

11. References:

- font size 11;

- NOT numerated section heading, capital letter, centred;

- all references must be referred in the text, e. g. [6],  [7-11], etc.;

- the full references should be listed at the end of the paper (justified starting with a number e. g. [1]) in numerical order of citation in the text;

- for references having two or more authors, names of all authors should be given;

- the references of Journals, Proceedings (Transactions, Book of abstracts, Proceedings on CD, etc.), Thesis, Books, Chapters, Patents and Literature or data on web sites and documents without authors are explained in the example above;

- only Latin alphabet must be used in References.

12. Figures:

- all figures must be referred in the text, e. g. Fig.1;

- the presented schemes must be visible and the inscriptions therein must be well readable;

- figure captions: below figures, left alignment, font size 10;

- number figures consecutively in accordance with their appearance in the text.

13. Tables:

- all tables must be referred in the text, e. g. Tab.3;

- table captions: above tables, left alignment with the table, font size 10;

- tables should be submitted as editable text and not as images;

- number tables consecutively in accordance with their appearance in the text;

- ensure that the data presented in tables do not duplicate results described elsewhere in the article;

- avoid the use of table constructions (have a look at the example above).

14. Equations:

- all equations and expressions should be numbered in parentheses, e. g. (1) with right alignment;

- equations and expressions should be referred within the text with corresponding number in parentheses, e. g. Eq.(5);

- all the variables which appear in the equations should be explained after that.

15. Header:

- font size 9;

- italic;

- centred;

- initials of the first author et al., followed by the title of the article;

- presented at each page (except for the first page).

16. Footer:

- font size 9;

- the following text should be used: © 2019 Bulgarian Academy of Sciences, Union of Chemists in Bulgaria;

- only on the first page;

- left alignment.

Manuscripts are submitted in English by e-mail. Articles which do not comply with this template will not undergo reviewing and publishing. Please consult the Guest editors for any questions you have.